| Safe System of Work |  |
| --- | --- |

| Company name | Elite Services Holdings Ltd | Department | Service |
| --- | --- | --- | --- |

| Contract | Customers Site | Prepared By | Gareth Lewis |
| --- | --- | --- | --- |

| Date prepared | 01/05/2025 | Review date | 01/05/2026 |
| --- | --- | --- | --- |

1. DESCRIPTION OF WORK

The Installation and service of: Soap, Hand Stz, Safe Seat, Sanitizer Dispensers and Skin Safety Centres

2. PERSONNEL COMPETENCE REQUIRMENTS

All Personnel to be fully trained.

3. SUPERVISORS DUTIES AND RESPONSIBILITIES

To assess the work to be carried out and ensure that all personnel required have been fully trained and carry the correct PPE.

4. MEANS OF ACCSESS

Access is required to all areas equipment is located. Sanitizers may need to be fitted behind panels.

5. MACHINERY, PLANT AND EQUIPMENT

Battery Drill, Cable & Stud Detector, Panel Lifters If Needed

6. LIFTING OPERATIONS AND EQUIPMENT

Remove any panels with Panel Lifters using the Safe Working System.

7. MATERIALS STORAGE, SECURITY AND HANDLING

8. SEQUENCE OF OPERATIONS

- Ensure the area is closed off and ‘2 meter’ social distancing rule is possible - Before Installation, ask the customer if there is any Asbestos on site, if so, ask for a risk assessment and plan of where the asbestos is located. You MUST NOT drill if there is any Asbestos in the vicinity of the install. - Close off the area of install, if this is not possible erect warning signs and restrict access in the specific area of work. - Determine the best position for the dispenser. If the dispensers need to be fitted behind panels, remove with panel lifters using the Safe Working System for lifting panels. - Using a cable and Stud detector check the area for hidden electricity cables, water/gas pipes and metal reinforcing, if found find an alternative position. - Using the Safe Working System for the use of battery operated drills, drill holes to the required depth. - Fit and fill the dispenser as per the manufactures instructions. - Safely dispose of any waste materials and clean down all surfaces of brick dust - Re-open area - Remove gloves, wash/sanitize hands

9.PERONAL PROTECTIVE EQUIPMENT

Latex gloves for servicing, Safety Footwear, Safety Glasses for Drilling

10. PROTECTION OF THIRD PARTIES

Close of area and erect warnings signs

11.ENVIROMENTAL ISSUES

All waste is to be removed from site and disposed of in accordance with the Environmental Protection Act 1990 and Elite Services Holdings company policy.

12. FIRST AID AND EMERGENCIES

All incidents and accidents are to be reported to the customer in the first instance and then to Elite Healthcare. All ‘Elite Services Holdings’ service personnel carry 1st aid kits on there vehicles. All engineers must carry mobile phones whilst working alone.

13. OTHER INSTRUCTIONS

N/A