SAFETY DATA SHEET – 70% Alcohol Hand Gel

1.  IDENTIFICATION OF THE PREPARATION AND THE COMPANY

# SUBSTANCE NAME:   70% Alcohol Hand Gel with moisturiser

SYNONYMS, TRADE NAMES:    Prosan, PN601, PN602, PN604

COMPANY IDENTIFICATION:  Maclin Sourcing Solutions Limited, Unit A3, Risby Business Park, Newmarket Road, Risby, Suffolk, IP28 6RD UK.

TELEPHONE: 00 44 (0) 1284 810 887 (office hours)

2.  COMPOSITION/INFORMATION ON INGREDIENTS:

INGREDIENT NAME		CONTENTS ( V/V)	   	               HEALTH RISK

CAS No		(class)  (R.No.)

Aqua					  33%					None

Ethyl Alcohol(alcohol denatured)		>  65%		64-17-5			R11

Carbomer				<   1%					None

Glycerine				<1%		000056-81-5		None

3.  HAZARDS IDENTIFICATION:

HIGHLY FLAMMABLE

4.  FIRST AID MEASURES:

FIRST AID:    	In the event of:

Swallowing:			Discourage vomiting.  Give plenty of milk or water to drink.  					Obtain immediate medical attention.

Inhalation of spray vapours:	Remove affected person to fresh air and seek medical advice

in the case of prolonged effect.

Eye Contact:			Wash thoroughly with clean water and seek immediate 					medical attention.

Skin Contact:			Wash thoroughly with clean water.  Seek medical attention for

persistent irritation.

Splashes on clothing:		Remove contaminated clothing.  Wash affected area of

skin as above.

5. FIRE FIGHTING MEASURES:

HIGHLY FLAMMABLE.  Use Foam, Powder, CO2, Halon or Sand to extinguish.

DO NOT USE WATER

6.  ACCIDENTAL RELEASE  MEASURES:

SPILL CLEANUP METHODS:

Small spillage may be diluted to waste with water.

For spillage's of greater than 10 litres use absorbent, non-combustible material then place in

suitable containers. Containers to be collected by waste disposal contractors.

7.  HANDLING & STORAGE:

USAGE PRECAUTIONS:

Avoid direct and prolonged contact with skin.

Avoid contact with eyes.  Avoid sources of ignition.

Do not smoke when handling this product.  Store away from extremes of heat.

Store away from sources of ignition.

8.  EXPOSURE CONTROLS AND PERSONAL PROTECTION:

INGREDIENT NAME				STD		LT EXP	ST EXP

( 8 hrs)		(10 min)

Aqua						No standard

Ethyl Alcohol					OES		1900mgm-3

Carbomer					No standard

Glycerine					No standard

9.  PHYSICAL AND CHEMICAL PROPERTIES:

Appearance:			Clear Viscous liquid

Odour				Alcohol

Solubility			Soluble in water

Solubility Value			N/A

Specific Gravity			0.9 +/- 0.01  g/ml nominal

Ph				7.0

Flash point			< 21 degrees C

10.  STABILITY AND REACTIVITY:

Materials to Avoid:    			Strong Oxidising Agents

Hazardous decomposition products:	Fire creates CO2 and CO.

11.  TOXICOLOGICAL INFORMATION:

TOXIC DOSE (LC50)  N/A

TOXIC DOSE (LD50)  > !0,000 mg/kg - rat

12.  ECOLOGICAL INFORMATION:

Current information suggests that this product is  non-biodegradable but will evaporate to give negligible residual.

Detergents in this product are biodegradable.

13.  DISPOSAL CONSIDERATIONS:

DISPOSAL METHODS:

Small quantities may be diluted to waste.

Large quantities - Contact local waste disposal companies.

14.  TRANSPORT INFORMATION:

UN No:  1170

ADR CLASS No: 3

AIR TRANSPORT CLASS No:

AIR SUB CLASS:

AIR PACK GR.:

HASCHEM CODE:

SEA TRANSPORT CLASS No:

SEA PACK GR.: III

RAIL TRANSPORT CLASS No:

CEFIC TEC (R) No:

UK LABEL FOR CONVEYANCE:  F,

15.  REGULATORY INFORMATION:

LABEL FOR SUPPLY:   F:  FLAMMABLE

RISK PHRASES;

R-11	  Highly Flammable

SAFETY PHRASES:

S-7		Keep container tightly closed.

S-16		Keep away from sources of ignition.  NO SMOKING.

S-2		Keep out of reach of children.

S26		In case of contact with eyes, rinse immediately with water. Seek 				medical advice

16.  OTHER INFORMATION:

NOTE:  The information herein is presented in good faith and believed to be correct at the date prepared however no warranty or representation expressed or implied is made as to the accuracy or completeness of the foregoing data and safety information or continuing accuracy of this data, Information is supplied on the condition that the persons receiving same will make their own determination as to the completeness and accuracy, safety and suitability for their purposes prior to use.  In no event will Maclin Group be responsible for damages of any nature whatsoever resulting from the use or reliance of this data. The user assumes all liability for any damage or injury resulting from abnormal use, from any failure to adhere to recommended practices or from any hazards inherent in the nature of the product.  END

Revision 4: 17th Feb 2015  LAST  UPDATED : 01/07/12  END   Revision 3.