| Safe System of Work |  |
| --- | --- |

| Company name | Elite Services Holdings Ltd | Department | Service |
| --- | --- | --- | --- |

| Contract | Customers Site | Prepared by | Gareth Lewis |
| --- | --- | --- | --- |

| Date prepared | 01/05/2025 | Review date | 01/05/2026 |
| --- | --- | --- | --- |

1. DESCRIPTION OF WORK

The Installation, Service and Removal of Vending & Roller Towel Dispensers on customer’s premises.

2. PERSONNEL COMPETENCE REQUIREMENTS

All Personnel to be fully trained in Manual Handling techniques, Working at Heights and Safe Working Systems.

3. SUPERVISORS’ DUTIES AND RESPONSIBILITIES

To assess the work to be carried out and ensure that all personnel required have been fully trained and carry the correct PPE.

4. MEANS OF ACCESS

Access is required to all the areas where equipment is located.

5. MACHINERY, PLANT AND EQUIPMENT

Battery Operated Drills, Cable & Stud Detector

6. LIFTING OPERATIONS AND EQUIPMENT

Please refer to manual handling

7. MATERIALS STORAGE, SECURITY AND HANDLING

All Materials are to be kept on the vehicle until needed.

8. SEQUENCE OF OPERATIONS

Installation/Removal - Ensure the area is closed off and ‘2 meter’ social distancing rule is possible - Prior to installation ask the contact if there is any Asbestos in the site, if so ask to see a copy of the Risk assessment and Plan. You must not drill if there is any Asbestos in the vicinity of the install. - Close off the area of install, if this is not possible place warning signs and restrict access in the specific area of work. - Check the area the dispenser is to be fitted for electricity cables, water and gas pipes using the cable and stud detector, if any are found relocate the dispenser. - Install and set up the dispenser as per the manufactures instructions. - Safely dispose of any waste materials and clean down all surfaces. - Reopen area. Service - Close off the area for servicing, if this is not possible place warning signs and restrict access in the specific area of work. - Service & Replenish dispenser as per the manufactures instructions. - Reopen area - Remove gloves, wash/sanitize hands

9. PERSONAL PROTECTIVE EQUIPMENT

Safety Shoes, Safety Glasses for drilling, Safety Hat & High Visibility Vest if needed KN95 protective mask

10. PROTECTION OF THIRD PARTIES

Close of area and erect warnings signs

11. ENVIRONMENTAL ISSUES

All waste is to be removed from site and disposed of in accordance with the Environmental Protection Act 1990 and Elite Services Holdings company policy.

12. FIRST AID AND EMERGENCIES

All incidents and accidents are to be reported to the customer in the first instance and then to Elite. All ‘Elite Services Holdings’ service personnel carry 1st aid kits on there vehicles. All engineers must carry mobile phones whilst working alone.

13. OTHER INSTRUCTIONS

Follow the safe system of work for the use of Step Ladders and Battery Operated Drills