| Safe System of Work |  |
| --- | --- |

| Company name | Elite Services Holdings Ltd | Department | Service |
| --- | --- | --- | --- |

| Contract | Elite Waste Transfer Station | Prepared by | Gareth Lewis |
| --- | --- | --- | --- |

| Date prepared | 01/05/2025 | Review date | 01/05/2026 |
| --- | --- | --- | --- |

1. DESCRIPTION OF WORK

Unloading of offensive waste contained in tiger bags from service vehicle into a 240ltr, 360ltr or 770ltr Euro Yellow Bin (EYB)

2. PERSONNEL COMPETENCE REQUIREMENTS

All Personnel to be trained in safe manual handling techniques and our safe working systems.

3. SUPERVISORS’ DUTIES AND RESPONSIBILITIES

To assess the work to be carried out and ensure that all personnel required have been trained and wear the correct PPE for this system of work

4. MEANS OF ACCESS

Unlock storage compound gates to access the bin area. If an empty EYB isn’t safely accessible from the front of the compound then you should safely move any obstructing full bins out of the storage compound and place into the loading bay area ensuring you apply its foot brake if it has one. Employee’s must never navigate themselves between bins within the compound to avoid risk of crushing or injury.

5. MACHINERY, PLANT AND EQUIPMENT

Service Vehicle, Euro Yellow Bins (EYB)

6. LIFTING OPERATIONS AND EQUIPMENT

Manual Handling Techniques. Trolleys are available for transporting waste from the vehicle to the EYB if required Bins & trolleys should be pushed and not pulled to avoid tripping and injury.

7. MATERIALS STORAGE, SECURITY AND HANDLING

Waste Storage Compound is padlocked. Euro Yellow Bins have Slam-lock lids and foot brakes.

8. SEQUENCE OF OPERATIONS

- The service vehicle should be parked wholly in a parking bay within the loading area. - Following means of access (4), safely wheel the empty EYB to your service vehicle and apply its foot brake. - Unlock the bin lid with your key and open its lid fully so it hangs down vertically to the rear of bin. - Remove bags one at a time from vehicle and place into the EYB. Ensure bags are handled by the top knot and away from the body to avoid needle stick injury. - Assistance should be sought for unloading larger quantities of waste. - Fill the EYB to the top. Do not overfill as to prevent the lockable lid from closing. - Close the bin lid ensuring that the slamlock is fixed to ensure waste is safely sealed in the EYB - Release the bin brake and safely wheel the bin to its storage area, Repeat previous steps if a 2nd bin is required. - (If applicable) Safely return any other bins that may have been moved out of the storage compound. - Close any gates that have been opened and lock the compound - Remove gloves, wash/sanitize hands.

9. PERSONAL PROTECTIVE EQUIPMENT

Safety Shoes, Latex Gloves, Hi Viz Vest are mandatory at all times.

10. PROTECTION OF THIRD PARTIES

Ensure that the vehicle is not blocking fire escape routes or pedestrian routes. Permission from a manager must first be attained before any doors or gates are wedged open.

11. ENVIRONMENTAL ISSUES

All waste is to be removed & disposed of in accordance with the Environmental Protection Act 1990 and Elite Services Holdings company policy.

12. FIRST AID AND EMERGENCIES

All incidents and accidents are to be reported to a supervisor or first aider within the first instance, then to a manager. All ‘Elite Services Holdings’ service personnel to carry 1st aid kits on their vehicles. All engineers must carry mobile phones whilst working alone for emergencies.

13. OTHER INSTRUCTIONS

Any bags that are excessively filled within a vehicle can be split into 2 bags, only if safe to do so.